# Project 1 – Queue Simulation Report

ENEL 573 L01 – Computer Networks, Fall 2026

Name: Temuulen Ankhbayar          Student ID: 30188632

Date: October 16, 2026

# Question 1 – Generating exponential random variables

With 1000 samples and λ = 75, the sample mean came out 0.78% from the theoretical 1/λ and the variance 6.08% from 1/λ², which is within normal sampling error for n = 1000.

## Derivation (inverse-transform method)

The CDF of an exponential random variable X with rate λ is:

F(x) = P(X ≤ x) = 1 − e−λx,   x ≥ 0

Set F(x) equal to a uniform random variable U ~ U(0,1) and solve for x:

1 − e−λx = U   ⇒   e−λx = 1 − U   ⇒   −λx = ln(1 − U)

x = −(1/λ) · ln(1 − U)

So each sample is X = −ln(1 − U)/λ. Since random() returns values in [0, 1), 1 − U is never 0, so the logarithm is always defined.

Theoretical mean and variance. With the pdf f(x) = λe−λx, integration by parts gives E[X] = ∫ x·λe−λx dx (0 to ∞) = 1/λ and E[X²] = ∫ x²·λe−λx dx (0 to ∞) = 2/λ², so Var[X] = E[X²] − (E[X])² = 2/λ² − 1/λ² = 1/λ².

Theoretical values for λ = 75:

E[X] = 1/λ = 1/75 = 0.013333          Var[X] = 1/λ2 = 1/5625 = 0.00017778

## Code (ENEL573_Project1_Q1.py)

Screenshot 1. ENEL573_Project1_Q1.py – exponential generator using the inverse method.

## Output of one run

Screenshot 2. Output of python ENEL573_Project1_Q1.py.

The sample mean and (unbiased) sample variance of the n = 1000 generated samples x₁ … xₙ are calculated as:

x̄ = (1/n) · Σ xᵢ          s² = (1/(n − 1)) · Σ (xᵢ − x̄)²

| Quantity | Simulated (n = 1000) | Theory | Error |
| --- | --- | --- | --- |
| Mean (s) | 0.013229 | 1/75 = 0.013333 | 0.78% |
| Variance (s²) | 0.00018858 | 1/75² = 0.00017778 | 6.08% |

The error is calculated as |simulated − theory| / theory × 100%:

Mean: |0.013229 − 0.013333| / 0.013333 × 100% = 0.78%

Variance: |0.00018858 − 0.00017778| / 0.00017778 × 100% = 6.08%

Both values are close to the theoretical ones, so the 1000 generated samples agree with an exponential random variable with λ = 75.

## Why the results are reasonable

The uniform generator passes the basic check first: mean 0.4948 vs 0.5 and variance 0.0851 vs 0.0833.

With only 1000 random samples, small differences from theory are expected: the mean is within 1% and the variance within about 6% of the theoretical values, and both get closer as the number of samples grows.

The seed changes each run, so the exact numbers differ from run to run but stay this close to theory.

# Question 2.1 – The M/M/1 discrete event simulator

The simulator pre-generates every arrival, departure and observer event in [0, T], sorts them by time, and walks the list once while updating three counters; E[N] and P_IDLE come from what the observers see.

## Variables

| Symbol | Meaning | Value used |
| --- | --- | --- |
| λ | Average packet arrival rate (packets/s) | ρC/L, from 20.8 to 79.2 |
| L | Average packet length (bits) | 12000 |
| C | Link transmission rate (bits/s) | 1 000 000 |
| ρ | Utilization = λL/C | 0.25 to 0.95, step 0.1 |
| α | Observer rate (events/s) | 5λ (manual asks for at least 3λ) |
| T | Simulation time (s) | starts at 1000, grows until stable |
| Na, Nd, No | Counters: arrivals, departures, observations | – |

## How it works

Arrivals. Inter-arrival times are exponential with rate λ (the Question 1 generator), added up until the time passes T.

Departures. Each arriving packet gets an exponential length with mean L, so its service time is length / C. If the server is busy, the packet starts when the previous one leaves; otherwise it starts on arrival: departure = max(arrival, previous departure) + service time. Departures after T are dropped.

Observers. A separate Poisson stream with rate α = 5λ.

Sort. All events go into one list sorted by time; this list is the DES.

Process. Each event is handled in order. Arrival → Na += 1. Departure → Nd += 1. Observer → No += 1, the packets in the system are Na − Nd, add that to a running sum, and if it is 0 count one idle observation. Walking the sorted list once with a for-loop does the same job as the manual’s “process the event, then delete it from the DES”: every event is used exactly once and never revisited.

## Performance metrics

E[N] = (1/No) · Σk=1..No (Na − Nd) at observer k

PIDLE = (number of observers that saw Na − Nd = 0) / No

In the code, last_departure holds the departure time of the previous packet, sum_packets adds up Na − Nd over all observer events, and idle_count counts the observer events that found Na − Nd = 0. simulate() returns sum_packets / No and idle_count / No.

## Stability check

As the manual describes, each ρ is run for T = 1000 s, then 2T, 3T, … and each result is compared with the previous run. A result is accepted once both E[N] and P_IDLE differ by less than 5% from the previous run two times in a row. The shortest possible stable T is therefore 3000 s. In the final run every ρ settled at 3000 s except ρ = 0.85, which needed 6000 s.

## Code (ENEL573_Project1_Q2.py)

Screenshot 3. ENEL573_Project1_Q2.py (part 1)

Screenshot 4. ENEL573_Project1_Q2.py (part 2)

Screenshots 3 and 4 show the whole file. Lines 71–104 loop over ρ = 0.25 … 0.95, print the results table and draw the two graphs with matplotlib.

# Question 2.2 – Results for L = 12000 bits, C = 1 Mbps

For ρ = 0.25 to 0.85 the simulator matches M/M/1 theory within 1.5% for E[N] and 1.4% for P_IDLE; at ρ = 0.95 the errors rise to 5.1% and 4.3%. For each ρ the arrival rate is set to λ = ρC/L, the stable run is taken, and the result is plotted against the theoretical curve.

## 1. E[N] as a function of ρ

Theory for M/M/1: E[N] = ρ/(1 − ρ).

Figure 1. E[N] vs ρ – simulation (blue) and theory (orange).

E[N] grows slowly up to ρ ≈ 0.7 and then sharply: going from ρ = 0.85 to 0.95 more than triples it (simulated 5.65 → 18.03; theory 5.67 → 19.0). The simulated curve sits right on the theoretical one up to ρ = 0.85 and falls slightly below it at ρ = 0.95. As ρ approaches 1 the server is almost never free, so packets pile up and the queue becomes unstable at ρ ≥ 1.

## 2. P_IDLE as a function of ρ

Theory for M/M/1: P_IDLE = P(N = 0) = 1 − ρ.

Figure 2. P_IDLE vs ρ – simulation (blue) and theory (orange).

P_IDLE falls in a straight line as ρ rises. This makes sense: ρ is the fraction of time the server is busy, so the rest of the time, 1 − ρ, it is idle. It is measured as the share of observers that found Na − Nd = 0.

## Accuracy and stability per run

| ρ | λ (packets/s) | E[N] error vs theory | P_IDLE error vs theory | Stable T (s) | Change vs previous T |
| --- | --- | --- | --- | --- | --- |
| 0.25 | 20.83 | 0.55% | 0.21% | 3000 | 1.44% |
| 0.35 | 29.17 | 0.50% | 0.04% | 3000 | 1.57% |
| 0.45 | 37.50 | 1.50% | 0.62% | 3000 | 0.53% |
| 0.55 | 45.83 | 1.32% | 0.40% | 3000 | 0.67% |
| 0.65 | 54.17 | 1.00% | 1.30% | 3000 | 0.70% |
| 0.75 | 62.50 | 0.13% | 0.08% | 3000 | 1.38% |
| 0.85 | 70.83 | 0.35% | 1.34% | 6000 | 0.90% |
| 0.95 | 79.17 | 5.12% | 4.27% | 3000 | 4.88% |

Errors are largest at ρ = 0.95 (5.1% for E[N], 4.3% for P_IDLE). Near saturation the queue length varies the most, so a longer run (larger T) would bring these values closer to theory. The “Change vs previous T” column is the larger of the E[N] and P_IDLE changes between the last two runs.

## Program output

Screenshot 5. Output of python ENEL573_Project1_Q2.py (simulated vs theoretical E[N] and P_IDLE for each ρ).

# Conclusion

The inverse-transform generator produces correct exponential samples, and the event-list simulator built on it reproduces M/M/1 theory for both E[N] and P_IDLE: within 1.5% for ρ ≤ 0.85 and within about 5% at ρ = 0.95.

Near saturation (ρ → 1) the queue length grows sharply and results need longer runs to stabilise, which is why ρ = 0.95 has the largest error.