ENEL 585 – Lab 3 Walkthrough

Single-phase and three-phase diode bridge rectifiers: step by step

This is a plain-language companion to the Lab 3 handout (ENEL585_Fall2026_Lab3_r1). It follows the handout's order: pre-lab, Part A (Experiments 1–2), Part B (Experiments 3–4), comparison, shutdown. All formulas come from Topic 10 (Rectifiers) and Topic 6 (AC Fundamentals). The handout and the TA always take priority over this sheet.

# 0. The big picture

You run four experiments on the SEMITEACH diode bridge, fed from the transformer box (36.7 V line-to-line, 60 Hz).

| Exp. | Bridge | DC side | What you're looking for |
| --- | --- | --- | --- |
| 1 | Single phase (A–B) | R = 12.5 Ω | Baseline: i_g should look like a sine, PF close to 1 |
| 2 | Single phase (A–B) | L–R (5 mH + 12.5 Ω) | Does 5 mH make i_d flat? (Pre-lab Q2 says no) |
| 3 | Three phase (A, B, C) | R = 12.5 Ω | Six-pulse v_d, i_ga in 120° blocks |
| 4 | Three phase (A, B, C) | L–R (5 mH + 12.5 Ω) | THD/PF vs 31 %/0.955, measure overlap u and L_c |

Rule for every experiment: transformer off → change wiring → transformer on → measure → transformer off.

You cannot repeat measurements after the session, so fill in the tables of Section 9 before you move on.

# 1. Before the lab: pre-lab answers

Given: V̂_g = √2 × 36.7 = 51.9 V (single phase), phase peak V̂_g = √2 × 21.2 = 30.0 V (three phase), R = 12.5 Ω, L = 5 mH, ω = 377 rad/s, ideal diodes.

| Q | Lecture formula | Result |
| --- | --- | --- |
| 1 | ⟨v_d⟩ = 2V̂_g/π; I_d = ⟨v_d⟩/R | ⟨v_d⟩ = 33.0 V, I_d = 2.64 A |
| 2 | X = 2ωL (ripple at 2ω); ¦Z¦ = √(R² + X²) | X = 3.77 Ω (X/R = 0.30), ¦Z¦ = 13.06 Ω ≈ R → 5 mH is NOT enough to hold i_d constant |
| 3a | ⟨v_d⟩ = (3√3/π)V̂_g; v_d from 1.5V̂_g to √3V̂_g | ⟨v_d⟩ = 49.6 V, I_d = 3.97 A, v_d: 45.0 to 52.0 V |
| 3b | u = cos⁻¹(1 − 2X_cI_d/(√3V̂_g)), X_c = ωL_c (one line) | X_c = 0.0377 Ω → u = 6.15° = 285 µs |
| 4 | k_p = cosθ · I_1/I_rms | cosθ = 1, but square-wave i_g has I_1 = 0.90I_rms → k_p = 0.90 |

Why the answers look the way they do:

I_d uses R, not |Z|: I_d is the dc (average) value. At dc the inductor's impedance jkωL is zero (k = 0); equivalently ⟨v_L⟩ = 0 in steady state.

Q2: at 120 Hz the inductor adds only 4 % to the impedance, so the 120 Hz ripple in v_d passes into i_d almost unchanged.

Q3 overlap: the 2 in ‘2X_cI_d’ already counts the two L_c's (line a and line c) in the commutation loop. Use X_c for one line.

Q4: harmonics carry no average power when v_g is sinusoidal but still raise I_rms, so S > P.

# 2. Safety and power rules (follow every time)

The 208 V cable plugs only into the back (HIGH VOLTAGE inputs). Never into front terminals A, B, C, N, G.

Before the ABB 208 V supply is turned on: both transformer breakers off.

Power on: high-voltage breaker first, then low-voltage breaker. Power off: low-voltage first, then high-voltage.

Between experiments, “transformer on/off” means the low-voltage breaker only. The HV breaker stays closed.

Every change (wiring, moving a probe, changing R, adding/removing L) is done with the transformer off. Never open the dc circuit or a load-bank switch while current flows.

Use the differential probes for all voltages. No scope ground lead anywhere in the circuit. G is not used.

Nothing on the SEMITEACH box except the rectifier terminals: no +15 V, no BNC, nothing on DC-link or inverter terminals.

TA sign-off before first power-on, and again after rewiring for Part B.

# 3. Part A setup: single-phase bridge

1.	Check both transformer breakers are off and the 208 V cable is in the back HIGH VOLTAGE inputs.

2.	Banana cables: transformer A → first Rectifier input (node a), transformer B → second Rectifier input (node b). Leave the third rectifier input, C, N and G unconnected.

3.	DC side, network (a): red DC output (+) → load bank → blue DC output (−). Close all 20 load-bank switches (20 × 250 Ω in parallel = 12.5 Ω).

4.	Ch 1 (differential, 10:1): + on rectifier input a, − on input b → measures v_ab.

5.	Ch 2 (differential, 10:1): across the DC rectifier outputs, + to + → measures v_d.

6.	Ch 3 (current probe, 100 mV/A, zeroed first): clamp on the line-a wire, arrow toward the rectifier → i_g.

7.	Ch 4 (current probe, 100 mV/A, zeroed first): clamp on the wire from the red DC output, arrow away from the rectifier → i_d.

8.	Set up the scope by hand (no Auto Scale), 5 ms/div: Ch 1 20 V/div, 0 V at centre; Ch 2 10 V/div, 0 V three divisions below centre; Ch 3 2 A/div, 0 A at centre; Ch 4 1 A/div, 0 A three divisions below centre. Trigger on v_ab, rising, 0 V, Auto.

9.	In the scope's channel menus, set the probe factors (10:1 for voltage, 100 mV/A for current) so readings come out in V and A.

STOP: TA checks the wiring and scope settings. Then close the HV breaker, then the LV breaker.

# 4. Experiment 1: single phase, R load

## Steps

1.	Transformer on (LV breaker). Wait a few seconds for the meter to settle.

2.	Capture v_ab, v_d, i_g, i_d over at least two 60 Hz periods. Save the screenshot.

3.	[Meas] on the scope (over a whole number of periods): ⟨v_d⟩ (mean of Ch 2), I_d (mean of Ch 4), min and max of i_d.

4.	Acuvim-L meter: [V/A] for V_AB and I_A, [P] for total P, hold [H]+[V/A] ~1 s → Harmonic for THD of i_g.

5.	Calculate PF = P/(V_AB · I_A) yourself. Do not use the meter's PF page in Part A (it refers to line-to-neutral voltage).

6.	Transformer off.

## What you should see

v_d = |v_ab|: humps from 0 to ~52 V, twice per cycle (120 Hz).

i_d = v_d/R: the same humps, dropping to ~0 at every zero crossing. i_g looks like a sine, in phase with v_ab.

Expect ⟨v_d⟩ a little below 33.0 V and I_d a little below 2.64 A, PF close to 1.

## Answering the questions

Compare with pre-lab Q1: % difference = (measured − 33.0)/33.0. Use the handout's accuracy figures (scope ±2.4 V at 10 V/div) to decide whether the gap is significant.

Why is ⟨v_d⟩ low? The pre-lab assumes ideal diodes and an ideal source. In reality two diodes conduct at a time (each drops roughly 1 V), and the transformer's L_c and winding resistance drop some voltage under load.

Where does the distortion in i_g come from (resistive load)? (1) The supply voltage itself is not a perfect sine (look at v_ab, often flat-topped), and i_g = v/R copies it. (2) Near each zero crossing the diodes need about two diode drops before they conduct, so i_g has a small flat gap at zero.

# 5. Experiment 2: single phase, L–R load

## Steps

1.	Transformer off. Insert the 5 mH inductor between the red DC output and the load bank (Fig. 2(b)). Ch 4 still clamps the wire from the red output.

2.	Transformer on. Capture v_ab, v_d, i_g, i_d over two periods.

3.	Record the same items as Exp. 1: ⟨v_d⟩, I_d, min/max of i_d, V_AB, I_A, P, THD of i_g.

4.	On the meter's Harmonic page, record the harmonics of i_g up to the 13th.

5.	Scope FFT on Ch 2 (v_d): read the 120 Hz component. Note whether the FFT shows rms or dBV, and convert to a peak value (peak = √2 × rms).

6.	Transformer off.

## What you should see

I_d about the same as Exp. 1 (the inductor does not change the average).

i_d is smoother and no longer touches zero, but still has a large 120 Hz ripple. i_g becomes a bit squarer, but nowhere near the ideal square wave.

THD rises above Exp. 1 but stays far below the ideal 48 %; PF stays well above 0.90.

## Answering the questions

Compare with class (constant i_d: THD 48 %, PF 0.90) and with Exp. 1. Your results sit between the two cases, much closer to Exp. 1. That agrees with pre-lab Q2: X = 3.77 Ω is too small next to R = 12.5 Ω to hold i_d constant.

How big would L need to be? One frequency at a time (Topic 6):

1.	Measured 120 Hz peak of v_d: call it V̂_d2.

2.	120 Hz current ripple: Î_d2 = V̂_d2 / |R + j2ωL|.

3.	Load-voltage ripple, peak-to-peak: Δv_R ≈ 2R·Î_d2. Require Δv_R ≤ 0.10·⟨v_d⟩.

4.	Solve: |Z| ≥ 20·R·V̂_d2 / ⟨v_d⟩, then L = √(|Z|² − R²) / (2ω).

Tip: Example only (use your own measured numbers): if V̂_d2 ≈ 22 V and ⟨v_d⟩ ≈ 33 V, then ¦Z¦ ≥ 167 Ω and L ≈ 0.22 H, about 44 times the 5 mH inductor. An inductor that size carrying 2.6 A dc is large, heavy and lossy, so it is not practical.

# 6. Part B: three-phase bridge

## Rewire

1.	Transformer off. Remove the inductor and restore network (a): red output → load bank → blue. All 20 switches closed.

2.	Add transformer C → third Rectifier input (node c).

3.	Move Ch 1: + on rectifier input a, − on the transformer N terminal → measures v_aN.

4.	Current probes stay where they are: Ch 3 now reads i_ga, Ch 4 still reads i_d.

5.	Scope: Ch 1 to 10 V/div, 0 V at centre. Trigger on v_aN, rising, 0 V, Auto. Others as before.

STOP: TA checks the Part B wiring before you switch on.

## Experiment 3: three phase, R load

1.	Transformer on. Capture v_aN, v_d, i_ga, i_d over two periods.

2.	Scope: ⟨v_d⟩, I_d, min/max of i_d, and min/max of v_d.

3.	Meter: all three line voltages and line currents, total P, THD of i_ga, and the total PF from the PF page (allowed in Part B).

4.	Transformer off.

What you should see and how to answer:

v_d: six bumps per cycle (360 Hz), between about 45 V and 52 V; ⟨v_d⟩ a bit under 49.6 V, I_d a bit under 3.97 A. Same reasons for the gap as Exp. 1 (diode drops, L_c, source resistance).

Explain the shape of i_ga with “top diode of the highest phase, bottom diode of the lowest phase” (Topic 10). Top diodes D1/D2/D3 connect to a/b/c; bottom diodes D4/D5/D6 connect to a/b/c. One cycle:

| Interval (60° each) | Highest / lowest | Diodes on | v_d | i_ga |
| --- | --- | --- | --- | --- |
| 1 | a / b | D1, D5 | v_ab | +i_d |
| 2 | a / c | D1, D6 | v_ac | +i_d |
| 3 | b / c | D2, D6 | v_bc | 0 |
| 4 | b / a | D2, D4 | v_ba | −i_d |
| 5 | c / a | D3, D4 | v_ca | −i_d |
| 6 | c / b | D3, D5 | v_cb | 0 |

So i_ga is positive for 120°, zero for 60°, negative for 120°, zero for 60°. With a resistor, each 120° block has two bumps on top (it is v_d/R). Label these intervals on your capture.

## Experiment 4: three phase, L–R load, and commutation

1.	Transformer off. Insert the 5 mH inductor between the red output and the load bank.

2.	Transformer on. Take the same measurements as Exp. 3, plus the harmonics of i_ga up to the 13th.

3.	Commutation zoom: time base 100 µs/div (then finer). Ch 3 (i_ga) and Ch 4 (i_d) at 1 A/div, then 0.5 A/div. Trigger on i_ga, rising, level near I_d/2, Normal mode.

4.	Find the rising edge of i_ga. Use the most sensitive current scale that keeps the whole rise on screen.

5.	Put a horizontal cursor on the i_d trace (the level i_ga is heading for).

6.	Vertical cursors: t1 where i_ga leaves zero, t2 where i_ga first reaches the i_d level. Ignore the short overshoot/snap after the rise: that is reverse recovery of the outgoing diode, not overlap.

7.	Record Δt = t2 − t1 and save the capture. Transformer off.

Working out u and L_c from your capture:

u [degrees] = Δt × 60 Hz × 360°      (e.g. 285 µs → 6.15°)

From 2X_cI_d = √3V̂_g(1 − cos u):   L_c = √3V̂_g(1 − cos u) / (2ωI_d)

Use V̂_g = 30.0 V (or √2 × your measured V_aN), I_d from the scope, ω = 377 rad/s. Compare u with the pre-lab 6.15° (which assumed L_c = 100 µH).

What you should see and how to answer:

I_d ≈ the same as Exp. 3 (L does not change the average): compare with 3.97 A.

i_ga close to clean 120° blocks; THD close to 31 % and PF close to 0.955. Small differences come from the remaining i_d ripple and the overlap rounding the edges.

Harmonics, Exp. 2 vs Exp. 4: both waveforms have half-wave symmetry, so no even harmonics. Single phase (Exp. 2): 3, 5, 7, 9, 11, 13. Three phase (Exp. 4): 5, 7, 11, 13 only, no 3rd or 9th. Triplen currents would be in phase in all three lines and need a neutral to return through, and the bridge has no neutral connection. For ideal blocks, I_h ≈ I_1/h.

# 7. Comparison questions

Table: for Exp. 1–4 list ⟨v_d⟩, (i_d,max − i_d,min)/I_d, THD and PF (use the template in Section 9).

Rank by PF from your measurements, and explain each with the source-current shape: the closer i_g is to a sine in phase with v, the higher the PF. Expect the single-phase R load (near-sine i_g) at the top and the three-phase cases near 0.95 (120° blocks with gaps).

Exp. 1→2 vs Exp. 3→4: single phase, v_d swings 0–52 V at 120 Hz, so adding L changes i_d and i_g a lot (but not enough to flatten them). Three phase, v_d only swings 45–52 V at 360 Hz, where the inductor's reactance is 3× larger (11.3 Ω), so i_d is already nearly flat with R alone and adding L changes very little.

1 kW load from this 1.2 kVA transformer: choose the three-phase bridge. Rated line current ≈ 1200/(√3 × 36.7) ≈ 18.9 A. Three phase at 1 kW: I_line ≈ 1000/(0.955 × √3 × 36.7) ≈ 16.5 A (OK). Single phase loads only two lines: I ≈ 1000/(0.90 × 36.7) ≈ 30 A (overloaded). Three phase also gives higher ⟨v_d⟩, less ripple and better PF. A dc inductor is optional: it slightly flattens i_d, traded against size, cost, weight and copper loss.

# 8. Shutdown

1.	Open the low-voltage breaker, then the high-voltage breaker.

2.	Turn off the current probes.

3.	Disconnect wiring and return equipment as the TA instructs.

# 9. Recording tables (fill in during the lab)

## Scope readings

| Exp. | ⟨v_d⟩ [V] | I_d [A] | i_d min [A] | i_d max [A] | v_d min [V] | v_d max [V] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | — | — |
| 2 |  |  |  |  | — | — |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |

## Meter readings

| Exp. | V (V_AB or V_AN) [V] | I_A [A] | P total [W] | THD i [%] | PF |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | calc P/(V_AB·I_A) = |
| 2 |  |  |  |  | calc P/(V_AB·I_A) = |
| 3 |  |  |  |  | meter: |
| 4 |  |  |  |  | meter: |

Part B also (Exp. 3 and 4): V_AB ____  V_BC ____  V_CA ____   I_A ____  I_B ____  I_C ____

## Harmonics (% of fundamental)

| Order h | 3 | 5 | 7 | 9 | 11 | 13 |
| --- | --- | --- | --- | --- | --- | --- |
| Exp. 2 i_g |  |  |  |  |  |  |
| Exp. 4 i_ga |  |  |  |  |  |  |
| Ideal 1/h | 33 % | 20 % | 14 % | 11 % | 9 % | 8 % |

Exp. 2 FFT: 120 Hz component of v_d = ________ (peak)

Exp. 4 overlap: Δt = ________ µs   →   u = ________ °   →   L_c = ________ µH

# 10. Formula sheet (lecture formulas only)

| Quantity | Formula | Source |
| --- | --- | --- |
| Single-phase bridge average | ⟨v_d⟩ = 2V̂_g/π (with L_c: (2V̂_g/π)(1 − x), x = X_cI_d/V̂_g) | Topic 10 |
| Single-phase overlap | u = cos⁻¹(1 − 2X_cI_d/V̂_g) | Topic 10 |
| Three-phase bridge average | ⟨v_d⟩ = (3√3/π)V̂_g − (3/π)X_cI_d | Topic 10 |
| Three-phase overlap | u = cos⁻¹(1 − 2X_cI_d/(√3V̂_g)), X_c = ωL_c per line | Topic 10 |
| DC current | I_d = ⟨v_d⟩/R (⟨v_L⟩ = 0 in steady state) | Topic 10 |
| Impedance at harmonic k | Z_L = jkωL; solve each frequency separately | Topic 6 |
| THD | THD = √(I_rms² − I_1²)/I_1 | Topic 6 / 10 |
| Power factor | k_p = P/(V_rms I_rms) = cosθ · I_1/I_rms = cosθ/√(1 + THD²) | Topic 6 |
| Square-wave current | I_1 = (2√2/π)I_d = 0.90I_d, I_h = I_1/h (odd h) | Topic 10 |